conversion between all four number system of integer and floating point values:

## Number System Conversion Methods

A number N in base or radix b can be written as:

(N)b = dn-1 dn-2 -- -- -- -- d1 d0 . d-1 d-2 -- -- -- -- d-m

In the above, dn-1 to d0 is the integer part, then follows a radix point and then d-1 to d-m is the fractional part. 

dn-1 = Most significant bit (MSB) 
d-m = Least significant bit (LSB)

Base in Number System

### 1. Decimal to Binary Number System Conversion

For Integer Part:

- Divide the decimal number by 2.

- Record the remainder (0 or 1).

- Continue dividing the quotient by 2 until the quotient is 0.

- The binary equivalent is the remainders read from bottom to top.

For Fractional Part:

- Multiply the fractional part by 2.

- Record the integer part (0 or 1).

- Take the fractional part of the result and repeat the multiplication.

- Continue until the fractional part becomes 0 or reaches the desired precision.

- The binary equivalent is the integer parts recorded in sequence.

Example: (10.25)10

Decimal to Binary Conversion

For Integer Part (10):

- Divide 10 by 2 → Quotient = 5, Remainder = 0

- Divide 5 by 2 → Quotient = 2, Remainder = 1

- Divide 2 by 2 → Quotient = 1, Remainder = 0

- Divide 1 by 2 → Quotient = 0, Remainder = 1

Reading the remainders from bottom to top gives 1010.

For Fractional Part (0.25):

- Multiply 0.25 by 2 → Result = 0.5, Integer part = 0

- Multiply 0.5 by 2 → Result = 1.0, Integer part = 1

The fractional part ends here as the result is now 0. Reading from top to bottom gives 01.

Thus, the binary equivalent of (10.25)10 is (1010.01)2.

### 2. Binary to Decimal Number System Conversion

For Integer Part:

- Write down the binary number.

- Multiply each digit by 2 raised to the power of its position, starting from 0 (rightmost digit).

- Add up the results of these multiplications.

- The sum is the decimal equivalent of the binary integer.

For Fractional Part:

- Write down the binary fraction.

- Multiply each digit by 2 raised to the negative power of its position, starting from -1 (first digit after the decimal point).

- Add up the results of these multiplications.

- The sum is the decimal equivalent of the binary fraction.

Example: (1010.01)2

1x23 + 0x22 + 1x21+ 0x20 + 0x2 -1 + 1x2 -2 = 8+0+2+0+0+0.25 = 10.25

Thus, (1010.01)2 = (10.25)10

### 3. Decimal to Octal Number System Conversion

For Integer Part:

- Divide the decimal number by 8.

- Record the remainder (0 to 7).

- Continue dividing the quotient by 8 until the quotient is 0.

- The octal equivalent is the remainders read from bottom to top.

For Fractional Part:

- Multiply the fractional part by 8.

- Record the integer part (0 to 7).

- Take the fractional part of the result and repeat the multiplication.

- Continue until the fractional part becomes 0 or reaches the desired precision.

- The octal equivalent is the integer parts recorded in sequence.

Example: (10.25)10

For Integer Part (10):

- Divide 10 by 8 → Quotient = 1, Remainder = 2

- Divide 1 by 8 → Quotient = 0, Remainder = 1

Octal equivalent = 12 (write the remainder, read from bottom to top). So, the octal equivalent of the integer part 10 is 12.

For Fractional Part (0.25):

- Multiply 0.25 by 8 → Result = 2.0, Integer part = 2

The fractional part ends here as the result is now 0. So, the octal equivalent of the fractional part 0.25 is 0.2.

The octal equivalent of (10.25)10 = (12.2)8

### 4. Octal to Decimal Number System Conversion

For Integer Part:

- Write down the octal number.

- Multiply each digit by 8 raised to the power of its position, starting from 0 (rightmost digit).

- Add up the results of these multiplications.

- The sum is the decimal equivalent of the octal integer.

For Fractional Part:

- Write down the octal fraction.

- Multiply each digit by 8 raised to the negative power of its position, starting from -1 (first digit after the decimal point).

- Add up the results of these multiplications.

- The sum is the decimal equivalent of the octal fraction.

Example: (12.2)8

1 x 81 + 2 x 80 +2 x 8-1 = 8+2+0.25 = 10.25

Thus, (12.2)8 = (10.25)10

### 5. Decimal to Hexadecimal Conversion

For Integer Part:

- Divide the decimal number by 16.

- Record the remainder (0-9 or A-F).

- Continue dividing the quotient by 16 until the quotient is 0.

- The hexadecimal equivalent is the remainders read from bottom to top.

For Fractional Part:

- Multiply the fractional part by 16.

- Record the integer part (0-9 or A-F).

- Take the fractional part of the result and repeat the multiplication.

- Continue until the fractional part becomes 0 or reaches the desired precision.

- The hexadecimal equivalent is the integer parts recorded in sequence.

Example: (10.25)10

Integer part:

- 10 ÷ 16 = 0, Remainder = A (10 in decimal is A in hexadecimal)

Hexadecimal equivalent = A

Fractional part:

- 0.25 × 16 = 4, Integer part = 4

Hexadecimal equivalent = 0.4

Thus, (10.25)10 = (A.4)16

### 6. Hexadecimal to Decimal Conversion

For Integer Part:

- Write down the hexadecimal number.

- Multiply each digit by 16 raised to the power of its position, starting from 0 (rightmost digit).

- Add up the results of these multiplications.

- The sum is the decimal equivalent of the hexadecimal integer.

For Fractional Part:

- Write down the hexadecimal fraction.

- Multiply each digit by 16 raised to the negative power of its position, starting from -1 (first digit after the decimal point).

- Add up the results of these multiplications.

- The sum is the decimal equivalent of the hexadecimal fraction.

Example: (A.4)16

(A × 160) + (4 × 16-1) = (10 × 1) + (4 × 0.0625)

Thus, (A.4)16 = (10.25)10

### 7. Hexadecimal to Binary Number System Conversion

To convert from Hexadecimal to Binary:

- Each hexadecimal digit (0-9 and A-F) is represented by a 4-bit binary number.

- For each digit in the hexadecimal number, find its corresponding 4-bit binary equivalent and write them down sequentially.

Example: (3A)16

- (3)16 = (0011)2

- (A)16 = (1010)2

Thus, (3A)16 = (00111010)2

### 8. Binary to Hexadecimal Number System Conversion

To convert from Binary to Hexadecimal:

- Start from the rightmost bit and divide the binary number into groups of 4 bits each.

- If the number of bits isn't a multiple of 4, pad the leftmost group with leading zeros.

- Each 4-bit binary group corresponds to a single hexadecimal digit.

- Replace each 4-bit binary group with the corresponding hexadecimal digit.

Example: (1111011011)2

0011 1101 1011
| | |
3 D B

Thus, (001111011011 )2 = (3DB)16

### 9. Binary to Octal Number System

To convert from binary to octal:

- Starting from the rightmost bit, divide the binary number into groups of 3 bits.

- If the number of bits is not a multiple of 3, add leading zeros to the leftmost group.

- Each 3-bit binary group corresponds to a single octal digit.

- The binary-to-octal conversion for each 3-bit group is as follows:

| Octal | Binary Equivalent |
| --- | --- |
| 0 | 000 |
| 1 | 001 |
| 2 | 010 |
| 3 | 011 |
| 4 | 100 |
| 5 | 101 |
| 6 | 110 |
| 7 | 111 |

- Replace each 3-bit binary group with the corresponding octal digit.

Example: (111101101)2

111 101 101
| | |
7 5 5

Thus, (111101101)2 = (755)8

### 10. Octal to Binary Number System Conversion

To convert from octal to binary:

- Each octal digit (0-7) corresponds to a 3-bit binary number.

- For each octal digit, replace it with its corresponding 3-bit binary equivalent.

Example: (153)8

- Break the octal number into digits: 1, 5, 3

- Convert each digit to binary:

  - 1 in octal = 001 in binary

  - 5 in octal = 101 in binary

  - 3 in octal = 011 in binary

Thus, (153)8 = (001101011)2